# Lab ASSIGNMENT#01

Introduction to Computer Architecture

(EEE341: Introduction to Computer Architecture)

Instructor: Dr Mehmood Qureshi

| Student Name: |  | Dua Fatima |
| --- | --- | --- |
| Registration No: |  | FA23-BCS-129 |
| Department: |  | Computer Science |
| Submission Date: |  | 28th March, 2025 |

COMSATS University Islamabad

LAB 01:

Code:

We start by taking the given code and pasting it into a new file. The file is then saved so we can proceed to assemble it. This opens the execute tab as shown below:

The register values were all 0 initially and will now change in the registers tab, indicated by the green highlighted row, as we continue running each line.

### Table

| Step | Instruction | Register Modified | New Value |
| --- | --- | --- | --- |
| 1 | addi $t0, $zero, 10 | $t0 | 10 |
| 2 | addi $t1, $zero, 20 | $t1 | 20 |
| 3 | addi $t2, $zero, 5 | $t2 | 5 |
| 4 | add $t3, $t0, $t1 | $t3 | 30 |
| 5 | sub $t4, $t3, $t2 | $t4 | 25 |

When the line sw $t4, 0($t0) is executed, MARS throws an exception and automatically opens the Coprocessor 0 tab, which serves as the exception handler.

This line was supposed to take the value from the $t4 register and store it at memory address 10, since this address comes from the base register 0($t0), which is $t0 + 0. However, this does not happen.

In MIPS, one word equals 4 bytes. Every piece of data stored in memory occupies 4 bytes, meaning each memory slot is 4 bytes wide and the next slot's address begins after every 4 bytes. Slots start at addresses 0, 4, 8, 12, 16... (multiples of 4). MIPS only permits storing a word at the beginning of a slot, not somewhere in the middle of one.

Theoretically, this should store the value in $t4, which is 25 in decimal or 19 in hexadecimal, into memory address 10 using the sw instruction. Then the lw instruction would load the value from that memory address into the destination register $t5, giving it the value 25 in decimal or 19 in hexadecimal.

After that, the PC normally increments by 4 after every instruction to move to the next line. However, when j END executes, the PC does not increment by 4. Instead, it jumps directly to the address of the END label, skipping everything in between. As a result, the instruction addi $t6, $zero, 100 is never reached and the value in $t6 remains 0. The Program Counter then lands at nop.

Results: Write the final values of the following registers.(in decimal)

Register Final Value

| $t0 | 10 |
| --- | --- |
| $t1 | 20 |
| $t2 | 5 |
| $t3 | 30 |
| $t4 | 25 |
| $t5 | 0 |
| $t6 | 0 |

Lab Questions:

Here are the answers to all 6 questions:

Q1. Which instructions in the program are R-format instructions?

R-format instructions are those that use only registers with no immediate values or memory addresses. In this program, add and sub are R-format instructions, as they follow the format: opcode $rd, $rs, $rt.

Q2. Which instructions access memory?

Only two instructions access memory directly:

- sw $t4, 0($t0) — Store Word (writes to memory)

- lw $t5, 0($t0) — Load Word (reads from memory)

Q3. What is the effective address used by the instruction sw $t4, 0($t0)?

The effective address is calculated as: Base Register ($t0) + Offset = 10 + 0 = 10

So the effective address is 10, which is not a valid aligned address in MIPS (must be a multiple of 4), which is why MARS throws an exception.

Q4. What value is stored in memory by the sw instruction?

The sw instruction attempts to store the value held in $t4, which is 25 in decimal (0x19 in hexadecimal). However, since address 10 is not word-aligned, MARS throws an exception and the store does not actually complete.

Q5. What value is loaded into register $t5?

The lw instruction loads the value from memory address 0($t0) into $t5. The loaded value is 25 in decimal (0x19 in hexadecimal).

Q6. Why was the instruction addi $t6, $zero, 100 not executed?

Because the j END instruction executed before it. The jump instruction causes the Program Counter to skip directly to the END label instead of incrementing by 4 as normal. Since addi $t6, $zero, 100 sits between j END and the END label, it is completely bypassed and never executed, leaving $t6 at 0.

TASK 1:

Modify the program and change: sw $t4, 0($t0) to sw $t4, 4($t0)

Code:

Step 3: Calculate address

Formula:

Address = base + offset

Here:

- $t0 = 10

- offset = 4

So:

Address = 10 + 4 = 14

Step 4: Convert to real MARS address

Base address = 0x10010000
Final = 0x10010000 + 14
      = 0x1001000E

Step 5: Check in Data Segment

- Open Data Segment tab

- Look around address 0x1001000C / 0x10010010

- You’ll see value stored near there

Final Answer :

1. New Memory Address = 14 (0x1001000E in MARS)
2. Value Stored = 25

TASK 2:

Modify the register initialization values: $t0 = 15 $t1 = 8 $t2 = 3

Predict values :

Calculate manually:

$t3 = $t0 + $t1 = 15 + 8 = 23
$t4 = $t3 - $t2 = 23 - 3 = 20
$t5 = value loaded from memory = 20

The value of $t3 is the sum of the values of $t0 and $t1. Since, 15+8=23. $t3 = 23 in decimal which is 17 in hexadecimal.

$t4 is the subtraction of the values of $t2 and $t3. Since, 23-3=20. $t4=20 in decimal and 14 in hexadecimal.

$t5 should be the same as the value of $t4 theoretically which is 20 in decimal.

Now, we’re going to verify by running the program.

$t3 is 17 in hexadecimal verified.

$t4 is 14 in hexadecimal verified.

MARS gives exception while storing the value of $t4 at memory address 10 because it is not divisible by 4. The memory addresses are always multiples of 4 where the data is stored since a word is of 4 bytes. Therefore, the value of $t5 cannot be verified by this code.

TASK 3:

Remove the jump instruction: j END. Run the program again. What is the new value of $t6? Explain why this change occurred.

Since we want to observe the value of $t6, we update the effective address where $t4 is stored to 12, which is divisible by 4 and will execute without errors. This allows the program to proceed to the next line that gives us the value of $t6, rather than getting stuck at an exception on the previous store and load word instructions.

When the jump instruction is removed, the addi $t6, $zero, 100 instruction executes successfully and gives us the value of $t6, which is 100 in decimal or 64 in hexadecimal. Previously, with the jump instruction present, the program counter would bypass this instruction entirely and jump straight to END.